/* -*- mode: c++; coding: utf-8 -*- */

namespace Feel {

/** \page NaturalConvection Natural Convection in a heated Tank

\author Christophe Prud'homme

\tableofcontents

<br>

<hr>

<br>

\section Description Description

The goal of this project is to simulate the fluid flow under natural convection: the heated fluid circulates towards the low temperature under the action of density and gravity differences. Thie phenomenon is important in the sense it models evacuation of heat, generated by friction forces for example, with  a cooling fluid.

We shall put in place a simple convection problem in order to study the phenomenon without having to handle the difficulties of more complex domaines. We describe then some necessary transformations to the equations, then we define quantities of interest to be able to compare the simulations with different parameter values.

To study the convection, we use a model problem: it consists in a rectangular tank of height \f$1\f$ and width \f$W\f$, in which the fluid is enclosed :

<center>

<table border=0px>

<tr>

<td>\image html convection_geo.png</td>

</tr>

<tr>

<td><center>Geometry of the model</center></td>

</tr>

</table>

</center>

We wish to know the fluid velocity \f$\mathbf{u}\f$, the fluid pressure \f$p\f$ and fluid temperature \f$\theta\f$.

We introduce the adimensionalized Navier-Stokes and heat equations parametrized by the Grashof and Prandtl numbers. These parameters allow to describe the various regimes of the fluid flow and heat transfer in the tank when varying them.

The adimensionalized steady incompressible Navier-Stokes equations reads:

\f{equation*}

\begin{split}

\mathbf{u}\cdot\nabla \mathbf{u} +\nabla p -\frac{1}{\sqrt{\Gr}} \Delta \mathbf{u} &= \theta \mathbf{e}_2\\

\nabla \cdot \mathbf{u} & = 0\ \text{sur}\ \Omega\\

\mathbf{u} & = \mathbf{0}\ \text{sur}\ \partial \Omega

\end{split}

\f}

where \f$\Gr\f$ is the Grashof number, \f$\mathbf{u}\f$ the adimensionalized velocity and \f$p\f$ adimensionalized pressure and \f$\theta\f$ the adimensionalized temperature. The temperature is in fact the difference between the temperature in the tank and the temperature \f$T_0\f$ on boundary \f$\Gamma_1\f$.

The heat equation reads:

\f{equation*}

\begin{split}

\mathbf{u} \cdot \nabla \theta -\frac{1}{\sqrt{\Gr}\Pr} \Delta \theta &= 0\\

\theta &= 0\ \text{sur}\ \Gamma_1\\

\frac{\partial \theta}{\partial n} &= 0\ \text{sur}\ \Gamma_{2,4}\\

\frac{\partial \theta}{\partial n} &= 1\ \text{sur}\ \Gamma_3

\end{split}

\f}

where \f$\Pr\f$ is the Prandtl number.

\section Influence Influence of parameters

What are the effects of the Grashof and Prandtl numbers ? We remark

that both terms with these parameters appear in front of the \f$\Delta\f$

parameter, they thus act on the diffusive terms. If we increase the

Grashof number or the Prandtl number the coefficients multiplying the

diffusive terms decrease, and this the convection, that is to say the

transport of the heat via the fluid, becomes dominant. This leads also

to a more difficult and complex flows to simulate. The influence of the Grashof and Prandtl

numbers are different but they generate similar difficulties and flow

configurations. Thus we look only here at the influence of the Grashof

number which shall vary in \f$[1, 1e7]\f$.

<center>

<table border=0px>

<tr>

<td>\image html flow_grashof.png</td>

</tr>

<tr>

<td><center>Velocity norm with respect to  Grashof, \f$ Gr=100, 10000,

100000, 500000\f$.<br> \f$h=0.01\f$ and \f$\Pr=1\f$.</center></td>

</tr>

</table>

</center>

\section Quantities Quantities of interest

We would like to compare the results of many simulations with respect

to the Grashof defined in the previous section. We introduce two

quantities which will allow us to observe the behavior of the flow and

heat transfer.

\subsection Quantities_Mean Mean temperature

We consider first the mean temperature on boundary \f$\Gamma_3\f$

\f{equation*}

T_3 = \int_{\Gamma_3} \theta

\f}

This quantity should decrease with increasing Grashof because the

fluid flows faster and will transport more heat which will cool down

the heated boundary \f$\Gamma_3\f$. We observe this behavior on this

figure.

<center>

<table border=0px>

<tr>

<td>\image html temp_grashof.png</td>

</tr>

<tr>

<td><center>Mean temperature with respect to the Grashof number;<br>

\f$h=0.02\f$ with \f$\mathbb{P}_3\f$ Lagrange element for the velocity,

\f$\mathbb{P}_2\f$ Lagrange for the pressure and \f$\mathbb{P}_1\f$

Lagrange for the temperature.</center></td>

</tr>

</table>

</center>

\subsection Quantities_Flow Flow rate

Another quantity of interest is the flow rate through the middle of the

tank. We define a segment \f$\Gamma_f\f$ as being the vertical top

semi-segment located at \f$W/2\f$ with height \f$1/2\f$. The flow rate, denoted \f$\mathrm{D}_f\f$, reads

\f{equation*}

\mathrm{D}_f =  \int_{\Gamma_f} \mathbf{u} \cdot \mathbf{e}_1

\f}

where \f$\mathbf{e}_1=(1,0)\f$. Note that the flow rate can be negative or

positive depending on the direction in which the fluid flows.

As a function of the Grashof, we shall see a increase in the flow

rate. This is true for small Grashof, but starting at \f$1e3\f$ the flow

rate decreases. The fluid is contained in a boundary layer which is

becoming smaller as the Grashof increases.

<center>

<table border=0px>

<tr>

<td>\image html debit_grashof.png</td>

</tr>

<tr>

<td><center>Behavior of the flow rate with respect to the Grashof number; <br>\f$h = 0.02\f$,

\f$\mathbb{P}_3\f$ for the velocity, \f$\mathbb{P}_2\f$ for the pressure and

\f$\mathbb{P}_1\f$ for the temperature.</center></td>

</tr>

</table>

</center>

section Implementation Implementation

This application in implemented in

\c "feel/doc/manual/heatns/convection.cpp". The implementation solve

the full nonlinear problem using the nonlinear solver framework.

\section Numerical Numerical Schemes

\subsection Numerical_Stokes The Stokes problem

Consider the following problem,

\f{equation*}

\mbox{Stokes: }\left\{

\begin{array}{rcc}

-\mu\Delta\mathbf{u} +

\nabla p =

\mathbf{f}\\

\nabla\cdot\mathbf{u} = 0\\

\mathbf{u}|_{\partial \Omega} = 0

\end{array}

\right.

\f}

where \f$\Omega \subset \mathbb{R}^d\f$. There are no boundary condition

on the pressure. This problem is ill-posed, indeed we only control the

pressure through its gradient \f$\nabla p\f$. Thus if \f$(\mathbf{u},p)\f$ is

a solution, then \f$(\mathbf{u},p+c)\f$ is also a solution with \f$c\f$ any

constant. This comes from the way the problem is posed: the box is

closed and it is not possible to determine the pressure inside. The

remedy is to impose arbitrarily a constraint on the pressure, e.g. its

mean value is zero. In other words, we add this new equation to the

problem

\f{equation}

\int_\Omega p = 0

\f}

\note This is also true for the incompressible Navier-Stokes equations. We chose Stokes to simplify the exposure.

\subsection Numerical_Reformulation Reformulation

In order to impose the previous condition, we introduce a new

unknown, a Lagrange multiplier, \f$\lambda \in \mathbb{R} \f$ and modify

the incompressibility equation. Our problem reads now, find

\f$(\mathbf{u},p,\lambda)\f$ such

that

\f{equation*}

\mbox{Stokes 2: }\left\{

\begin{array}{rcl}

-\mu\Delta\mathbf{u} +

\nabla p &=&

\mathbf{f}\\

\nabla\cdot\mathbf{u} + \lambda &=& 0\\

\mathbf{u}|_{\partial \Omega} &=& 0\\

\int_\Omega p &=& 0

\end{array}

\right.

\f}

\note

The pressure field \f$p\f$ can actually be seen as a Lagrange multiplier

for the velocity \f$\mathbf{u}\f$ in order to enforce the constraint

\f$\nabla \cdot \mathbf{u} = 0\f$. \f$\lambda\f$ will play the same role but

for the pressure to enforce the condition. As \f$h \rightarrow 0\f$, \f$\lambda \rightarrow 0\f$ as well as the divergence of

\f$\mathbf{u}\f$. Note also that \f$\int_\Omega \nabla \cdot \mathbf{u}

\approx - \int_\Omega \lambda\f$ from the second equation.

\subsection Numerical_Variational Variational formulation

The variational formulation now reads: find \f$(\mathbf{u}, p,

\lambda) \in \mathbf{H}^1_0(\Omega) \times L^2_0(\Omega) \times

\mathbb{R}\f$ such that for all \f$(\mathbf{v}, q, \eta) \in

\mathbf{H}^1_0(\Omega) \times L^2_0(\Omega) \times \mathbb{R}\f$

\f{equation*}

\label{notes:eq:20}

\mbox{Stokes 3: }\left\{

\begin{array}{rcl}

\int_\Omega \Big(\nabla \mathbf{u} \colon \nabla \mathbf{v} + \nabla \cdot \mathbf{v} p\Big) &=& \int_\Omega \mathbf{f} \cdot \mathbf{v}\\

\int_\Omega \Big(\nabla\cdot\mathbf{u} q + \lambda q\Big)   &=& 0\\

\int_\Omega p \eta &=& 0

\end{array}

\right.

\f}

Summing up all three equations we get the following condensed formulation:

\f{equation*}

\label{notes:eq:19}

\int_\Omega \nabla \mathbf{u} \colon \nabla \mathbf{v} + \nabla \cdot \mathbf{v} p + \nabla \cdot \mathbf{u} q + \lambda q + \eta p = \int_\Omega \mathbf{f} \cdot \mathbf{v}

\f}

where \f$\mathbf{H}^1_0(\Omega)= \Big\{ \mathbf{v} \in \mathbf{L}^2(\Omega), \nabla \mathbf{v} \in [L^2(\Omega)]^{d\times d},\ \mathbf{v} = 0\ \text{on}\ \partial \Omega \Big\}\f$,

\f$L^2_0(\Omega)= \Big\{ v \in L^2(\Omega),\ \int_\Omega v = 0\Big\}\f$, and

\f$\mathbf{L}^2(\Omega)= \Big\{ \mathbf{v} \in [L^2(\Omega)]^d\Big\}\f$ that is to say each component of a  vector field of \f$\mathbf{L}^2(\Omega)\f$ are in \f$L^2(\Omega)\f$.

\subsection Numercial_Implementation Implementation

\co

//basis

typedef Lagrange<Order, Vectorial> basis_u_type; // velocity

typedef Lagrange<Order-1, Scalar> basis_p_type; // pressure

typedef Lagrange<0, Scalar> basis_l_type; // multipliers

typedef bases<basis_u_type, basis_p_type, basis_l_type> basis_type;

///space: product of the velocity, pressure and multiplier spaces

typedef FunctionSpace<mesh_type, basis_type, value_type> space_type;

// ...

space_ptrtype Xh = space_type::New( mesh );

element_type U( Xh, "u" );

element_type V( Xh, "v" );

element_0_type u = U.element<0>();

element_0_type v = V.element<0>();

element_1_type p = U.element<1>();

element_1_type q = V.element<1>();

element_2_type lambda = U.element<2>();

element_2_type nu = V.element<2>();

// ...

sparse_matrix_ptrtype D( M_backend->newMatrix( Xh, Xh ) );

form2( Xh, Xh, D, _init=true )=

integrate( elements(mesh), im,

// $\nabla \mathbf{u} \colon \nabla \mathbf{v}$

mu*trace(deft*trans(def))

// $\nabla \cdot \mathbf{v} p + \nabla \cdot \mathbf{u} q$

- div(v)*idt(p) + divt(u)*id(q)

// $\lambda q + \eta p$

+id(q)*idt(lambda) + idt(p)*id(nu) );

// ...

\eco

\subsection Numerical_Fic Fix point iteration for Navier-Stokes

\subsubsection Fix_Steady Steady incompressible Navier-Stokes equations

Consider the following steady incompressible Navier-Stokes

equations, find \f$(\mathbf{u},p)\f$ such that

\f{equation}

\label{notes:eq:7}

\begin{split}

\underbrace{\rho \mathbf{u} \cdot \nabla \mathbf{u}}_{\text{convection}} - \underbrace{\nu \Delta  \mathbf{u}}_{\text{diffusion}} + \nabla p &=  \mathbf{f} \ \text{on}\ \Omega \\

\nabla \cdot \mathbf{u} &= 0 \\

\mathbf{u} &= \mathbf{0}\ \text{on}\ \partial \Omega

\end{split}

\f}

where \f$\rho\f$ is the density of the fluid, \f$\nu\f$ is the dynamic

viscosity of the fluid(la viscosit	matique \f$\eta = \nu/\rho\f$) and \f$\mathbf{f}\f$ is the external force

density applied to the fluid, (e.g. \f$\mathbf{f}=-\rho g \mathbf{e}_2\f$ with \f$\mathbf{e}_2=(0,1)^T\f$ ).  This equation system is nonlinear due

to the \f$\mathbf{u} \cdot \nabla \mathbf{u}\f$ convection term. A

simple approach to solve \f$(2)\f$ is to use a fix point

algorithm.

The fixpoint algorithm for NS reads as follows, find

\f$(\mathbf{u}^{(k)},p^{(k)})\f$ such that

\f{equation*}

\label{notes:eq:13}

\begin{split}

\rho\mathbf{u}^{(k-1)} \cdot \nabla \mathbf{u}^{(k)} - \nu \Delta  \mathbf{u}^{(k)} + \nabla p^{(k)} &= \mathbf{f} \ \text{on}\ \Omega \\

\nabla \cdot \mathbf{u}^{(k)} &= 0 \\

\mathbf{u}^{(k)} &= 0\ \text{on}\ \partial \Omega\\

(\mathbf{u}^{(0)},p^{(0)}) &= (\mathbf{0},0)

\end{split}

\f}

This system  is now linear at each iteration \f$k\f$ and we

can write the variational formulation accordingly. A stopping

criterium is for example that

\f$\|\mathbf{u}^{k}-\mathbf{u}^{(k-1)}\|+\|p^{k}-p^{(k-1)}\| <

\epsilon\f$ where \f$\epsilon\f$ is a given tolerance (e.g. \f$1e-4\f$) and

\f$\|\cdot\|\f$ is the \f$L_2\f$ norm.

Here is the implementation using \Feel:

\co

// define some tolerance $\epsilon$

epsilon = 1e-4;

// set $(\mathbf{u}^{(0)},p^{(0)})$ to $(\mathbf{0},0)$

velocity_element_type uk(Xh);

velocity_element_type uk1(Xh);

pressure_element_type pk(Ph);

pressure_element_type pk1(Ph);

// by default uk1, uk and pk,pk1 are initialized to 0

// assemble the linear form associated to $\mathbf{f}$

// store in vector $F$, it does not change over the iterations

// iterations to find $(\mathbf{u}^{(k)},p^{(k)})$

do

{

// save results of previous iterations

uk1 = uk;

pk1 = pk;

//assemble for bilinear form  associated to

// $\rho\mathbf{u}^{(k-1)} \cdot \nabla \mathbf{u}^{(k)} - \nu \Delta  \mathbf{u}^{(k)} + \nabla p^{(k)}$

// store in matrix $A^{(k)}$

// solve the system $A^{(k)} X = F$ where $X = (\mathbf{u}^{(k)},p^{(k)})^T$

// use uk,uk1 and pk,pk1 to compute the error estimation at each iteration

error = $\|\mathbf{u}^{k}-\mathbf{u}^{(k-1)}\|+\|p^{k}-p^{(k-1)}\|$

} while( error > epsilon );

\eco

\subsection Nuerical_Coupling A Fix point coupling algorithm

\subsubsection Coupling_Problem Coupling fluid flow and heat transfer: problem

Recall that we have to solve two coupled problems :

\f[

\mbox{Heat(\textbf{u}) }\left\{

\begin{array}{rcc}

- \kappa\Delta T + \mathbf{u}\cdot\nabla T &=& 0 \\

T|_{\Gamma_1} &=& T_0 \\

\frac{\partial T}{\partial \mathbf{n}}|_{\Gamma_3} &=&1 \\

\frac{\partial T}{\partial \mathbf{n}}|_{\Gamma_2,\Gamma_4} &=& 0

\end{array}

\right.

\f]

and

\f[

\mbox{Stokes(T) : }\left\{

\begin{array}{rcc}

-\nu\Delta\mathbf{u} +

\frac{1}{\rho}\nabla p =

\mathbf{F}\\

\nabla\cdot\mathbf{u} = 0\\

\mathbf{u}|_{\partial \Omega} = 0

\end{array}

\right.

\f]

Where \f$\mathbf{F}\f$ can be taken as  \f$

\left(

\begin{array}{c}

0 \\

\beta(T-T_0)

\end{array}

\right)

\f$

for some \f$\beta > 0\f$. \f$\beta\f$ is called the \em dilatation \em coefficient.

\subsubsection Coupling_Algorithm Coupling fluid flow and heat transfer: algorithm

Here is a simple algorithm fix point strategy in pseudo-code:

\co

double tol = 1.e-6;

int maxIter = 50;

//Initial guess Un = 0

do

{

// Find Tn solution of Heat(Un)

// Find Unp1 solution of Stokes(Tn)

// compute stopTest = norme(Unp1 - Un)

// Un = Unp1

}while((stopTest < tol) && (niter <= maxIter));

\eco

\note

To solve the unsteady problems, one can insert the previous loop in

the one dedicated to time discretization

\subsection Numerical_Newton A Newton coupling algorithm

\subsubsection Newton_Scheme A fully coupled scheme

Another possiblity is to use a Newton method which allows us to

solve the full nonlinear problem coupling velocity, pressure and

temperature

\f{equation*}

\label{notes:eq:21}

\text{Find}\ X\ \text{such that}\ F(X) = 0

\f}

the method is iterative and reads, find \f$X^{(n+1)}\f$ such that

\f{equation}

\label{notes:eq:22}

J_F(X^{(n)})( X^{(n+1)}-X^{(n)}) = - F (X^{(n)})

\f}

starting with \f$X^{(0)} = \mathbf{0}\f$ or some other initial value and

where \f$J_F\f$ is the jacobian matrix of \f$F\f$ evaluated at

\f$X=((u_i)_i,(p_i)_i,(\theta_i)_i)^T\f$.  For any \f$\phi_k, \psi_l\f$ and

\f$\rho_m\f$ the \em test functions associated respectively to velocity,

pressure and temperature, our full system reads, Find \f$X=((u_i)_i,(p_i)_i,(\theta_i)_i)^T\f$ such that

\f{equation*}

\label{notes:eq:23}

\begin{array}{rll}

F_1((u_i)_i,(p_i)_i,(\theta_i)_i)&=\sum_{i,j} u_i u_j a(\phi_i,\phi_k,\phi_j) - \sum_i p_i b(\phi_k,\psi_i) + \sum_i \theta_i c(\rho_i, \phi_k)+\sum_i u_i d(\phi_i,\phi_k)  &= 0\\

F_2((u_i)_i,(p_i)_i,(\theta_i)_i)&=\sum_i u_i b(\phi_i,\psi_l) &=0\\

F_3((u_i)_i,(p_i)_i,(\theta_i)_i)&=\sum_{i,j} u_i\theta_j e(\phi_i,\rho_j,\rho_m) + \sum_i \theta_i f(\rho_i,\rho_m)-g(\rho_m) &=  0

\end{array}

\f}

where \f$F=(F_1,F_2,F_3)^T\f$ and

\f{equation*}

\label{notes:eq:26}

\begin{array}{rl}

a(\mathbf{u},\mathbf{v},\beta) &= \int_\Omega \mathbf{v}^T ((\nabla \mathbf{u} )\beta)\\

b(\mathbf{v},p) &= \int_\Omega p (\nabla \cdot \mathbf{v}) - \int_{\partial \Omega} \mathbf{v}\cdot\mathbf{n} p\\

c(\theta,\mathbf{v})&= \int_\Omega \theta \mathbf{e}_2 \cdot \mathbf{v}\\

d(\mathbf{u},\mathbf{v}) &= \frac{1}{\sqrt{\mathrm{Gr}}} \Big(\int_\Omega \nabla \mathbf{u} \colon (\nabla \mathbf{v})^T - \int_{\partial \Omega} ((\nabla \mathbf{u}) \mathbf{n})\cdot \mathbf{v}\Big)\\

e(\mathbf{u},\theta,\chi) &= \int_\Omega (\mathbf{u}\cdot \nabla \theta) \chi \\

f(\theta,\chi) &=\frac{1}{\sqrt{\mathrm{Gr}}\mathrm{Pr}} \Big( \int_\Omega \nabla \theta \cdot \nabla \chi - \int_{\Gamma_1} (\nabla \theta \cdot \mathbf{n} ) \chi \Big)\\

g(\chi) &=\frac{1}{\sqrt{\mathrm{Gr}}\mathrm{Pr}} \int_{\Gamma_3} \chi

\end{array}

\f}

\note

Note that the boundary integrals are kept in order to apply the

weak Dirichlet boundary condition trick, see next section.

\subsubsection Newton_Jacobian Jacobian matrix

In order to apply the newton scheme, we need to compute the jacobian

matrix \f$J_F\f$ by deriving each equation with respect to each

unknowns, ie \f$u_i, p_i\f$ and \f$\theta_i\f$.

Consider the first equation

\li Deriving the first equation with respect to \f$u_i\f$ we get

\f{equation*}

\label{notes:eq:301}

\frac{\partial F_1}{\partial u_i} = \sum_j u_j a(\phi_i,\phi_k,\phi_j) + \sum_i u_i a(\phi_i,\phi_k,\phi_j) + d(\phi_i,\phi_k)

\f}

\li Deriving the first equation with respect to \f$p_i\f$ we get

\f{equation*}

\label{notes:eq:302}

\frac{\partial F_1}{\partial p_i} =  -b(\phi_k,\psi_l)

\f}

\li Deriving the first equation with respect to \f$\theta_i\f$ we get

\f{equation*}

\label{notes:eq:303}

\frac{\partial F_1}{\partial \theta_i} = c(\rho_i,\rho_k)

\f}

Consider the second equation, only the derivative with respect to \f$u_i\f$ is non zero.

\f{equation*}

\label{notes:eq:31}

\frac{\partial F_2}{\partial u_i} = b(\phi_i,\psi_l)

\f}

Finally the third component

\li Deriving with respect to \f$u_i\f$

\f{equation*}

\label{notes:eq:33}

\frac{\partial F_3}{\partial u_i} = \sum_j \theta_j e(\phi_i,\rho_j,\rho_m)

\f}

\li Deriving with respect to \f$p_i\f$,

\f{equation*}

\label{notes:eq:34}

\frac{\partial F_3}{\partial p_i} = 0

\f}

\li Deriving with respect to \f$theta_i\f$,

\f{equation*}

\label{notes:eq:35}

\frac{\partial F_3}{\partial \theta_i} = \sum_j u_j e(\phi_j,\rho_i,\rho_m) + f(\rho_i,\rho_m)

\f}

\f{equation*}

\label{notes:eq:36}

J_F =

\begin{pmatrix}

\frac{\partial F_1}{\partial u_i} & \frac{\partial F_1}{\partial p_i} & \frac{\partial F_1}{\partial \theta_i} \\

{\frac{\partial F_2}{\partial u_i}} & {\frac{\partial F_2}{\partial p_i}}(=0) & {\frac{\partial F_2}{\partial \theta_i}}(=0) \\

\frac{\partial F_3}{\partial u_i} & {\frac{\partial F_3}{\partial p_i}}(=0) & \frac{\partial F_3}{\partial \theta_i}

\end{pmatrix}

\f}

In order to implement \f$J_F\f$ and solve \f$(3)\f$, \f$J_F\f$ can be

expressed as the matrix associated with the discretisation of

\f{equation*}

\label{notes:eq:37}

\begin{array}{rl}

a(\mathbf{u},\mathbf{v},\beta_1) + a(\beta_1, \mathbf{v}, \mathbf{u})+d(\mathbf{u},\mathbf{v})-b(\mathbf{v},p)+c(\theta,\mathbf{v}) &= 0\\

b(\mathbf{u},q)&=0\\

e(\beta_1,\theta,\chi)+f(\theta,\chi)+e(\mathbf{u},\beta_2,\chi)&=0\\

\end{array}

\f}

where \f$\beta_1 = u^{(n)}\f$, \f$\beta_2=\theta^{(n)}\f$ are known from the

previous Newton iteration, indeed \f$J_F\f$ is actually evaluated in \f$X^{(n)}\f$.

\subsubsection Newton_Implementation Implementation

Now we use the \feel non linear framework in order to implement our

Newton scheme.

We need to define two new functions in our application

\li \c updateJacobian(X,J) which takes as input \c X\f$=X^{(n)}\f$ and returns the matrix \c J=\f$J_F(X^{(n)})\f$

\li \c updateResidual(X,R) which takes as input \c X\f$=X^{(n)}\f$ and returns the vector \c R=\f$F(X^{(n)})\f$

\note

Only the PETSC backend supports the nonlinear solver framework.

Use  in the command line like in the first section

\co

--backend=petsc

\eco

Here is a snippet of code that implements the nonlinear framework.

\co

class MyApp

{

public:

void run();

void updateResidual( const vector_ptrtype& X, vector_ptrtype& R );

void updateJacobian( const vector_ptrtype& X, sparse_matrix_ptrtype& J);

void solve( sparse_matrix_ptrtype& D, element_type& u, vector_ptrtype& F );

private:

backend_ptrtype M_backend;

sparse_matrix_ptrtype M_jac;

vector_ptrtype M_residual;

};

void

MyApp::run()

{

// ...

// plug the updateResidual and updateJacobian functions

// in the nonlinear framework

M_backend->nlSolver()->residual = boost::bind( &self_type::updateResidual,

boost::ref( *this ), _1, _2 );

M_backend->nlSolver()->jacobian = boost::bind( &self_type::updateJacobian,

boost::ref( *this ), _1, _2 );

vector_ptrtype U( M_backend->newVector( u.functionSpace() ) );

*U = u;

vector_ptrtype R( M_backend->newVector( u.functionSpace() ) );

this->updateResidual( U, R );

sparse_matrix_ptrtype J;

this->updateJacobian( U, J );

solve( J, u, R );

*U = u;

this->updateResidual( U, R );

// R(u) should be small

std::cout << "R( u ) = " << M_backend->dot( U, R ) << "\n";

}

void

MyApp::solve( sparse_matrix_ptrtype& D, element_type& u, vector_ptrtype& F )

{

vector_ptrtype U( M_backend->newVector( u.functionSpace() ) );

*U = u;

M_backend->nlSolve( D, U, F, 1e-10, 10 );

u = *U;

}

void

MyApp::updateResidual( const vector_ptrtype& X, vector_ptrtype& R )

{

// compute R(X)

R=M_residual;

}

void

MyApp::updateJacobian( const vector_ptrtype& X, vector_ptrtype& R )

{

// compute J(X)

J=M_jac;

}

\eco

see \c bratu.cpp or \c nonlinearpow.cpp for example.

*/

}